RxANSI 1.10beta2: Combined ANSI and LESS driver. AMIS v3.6 compliant.

Public Domain by Henrik Haftmann and Christian Masloch

Purpose

-------

RxANSI is based on Henrik Haftmann's ANSI, a command line driver for ANSI

screen functions. It replaces ANSI.SYS of MS-DOS for all screen functions.

Keyboard functions are not implemented because they can be used to create

so-called "ANSI bombs" (key remapping sequences that result in undesired

actions for some keys, like "FORMAT C:" if you press enter) and writing the key

remapping is harder and would result in requiring more memory.

Information on the ANSI.SYS screen and keyboard functions can be obtained with

the command "HELP ANSI.SYS" in MS-DOS so I won't include a description here.

(Get documentation on the ANSI.SYS escape sequences somewhere else if you don't

use MS-DOS.)

RxANSI additionally has LESS functions: programs that display such sequences

(with backspace and underscore characters) now don't need to be piped through

LESS to display their colourful output. LESS "bold" text is displayed green,

LESS "underlined" text yellow.

Naturally RxANSI doesn't require much memory (currently about 900 bytes only)

hence it fits on any system. And even if that's too much, RxANSI can be

uninstalled. The output is completely done through the BIOS; though the BIOS

access is minimized. The output speed is halved compared to the speed without

any ANSI, but that's still much faster than with ANSI.SYS. (Look for NANSI.SYS

if you want speed. But you can't uninstall it, and it uses more memory than

RxANSI.)

Beside RxANSI's uninstall capability the /a and /l options can switch off (and

on) ANSI and LESS functions. This can be useful if RxANSI can't be uninstalled

because an AMIS incompatible TSR was installed after RxANSI.

An overview of the options can be obtained with the command "RXANSI /?", as

usual. More specific information for some of the options follows:

/c option

---------

By default, RxANSI links UMBs into the DOS memory chain if available and the

DOS memory allocation is set to search first in the UMBs. Hence the resident

code is installed into an UMB if there's a suitable free UMB found. Specifying

the /c option will instead insure that UMBs aren't linked into the DOS memory

chain: the resident code will be installed into a block of "conventional" or

low memory. (However the /l option is already used to control LESS functions.)

Note:	Don't load RxANSI with "LOADHIGH" or "LH". It could prevent RxANSI from

finding the best suitable UMB and doesn't help in any way.

/e option

---------

This "undocumented" option (not found in the /? help) prevents freeing RxANSI's

environment memory block before the block for the resident code is allocated.

The environment block will however not stay resident after RxANSI terminated.

The /e option may prevent RxANSI from finding the best usable memory block and

is intended for testing purposes only.

/t option

---------

The second undocumented option forces RxANSI to relocate the resident code to

the top of the allocated memory block. It doesn't force RxANSI to use the last

memory block available. The /t option is intended for testing purposes only.

Rx?

---

RxANSI isn't written for RxDOS only, and it possible doesn't run on the current

RxDOS version. However it's another free DOS program written in Assembly

language so I decided to use the "Rx" prefix to distinguish the new RxANSI from

both the former ANSI 1.01 and ANSI.SYS of MS-DOS.

Known bugs

----------

If LESS is switched on and the default DOS line editor is used, typeing either

blank or underscore then backslash is parsed as LESS sequence resulting in a

coloured cursor at this position until another character is written there. If

this troubles you, switch off LESS using the /l- option.

History

-------

RxANSI 1.10beta2, 2008-12-09, released by Christian Masloch

- /s option removed, interrupt 2Fh always hooked

- Doesn't answer to the ANSI.SYS installation check if ANSI switched off

RxANSI 1.10beta1, 2008-08-24, released by Christian Masloch

- Documentation and messages now English but the source still contains

alternative German messages

- AMIS v3.6 and IISP compliant; hooks interrupt 2Dh

- Installs high; relocates resident if required; doesn't leave PSP resident

- Optionally also hooks interrupt 2Fh

- New options /c, /s, /n, /e, /t; invalid options show help (removed /?, /h)

ANSI 1.01, 1994-10-28, released by Henrik Haftmann

- Bugfix: While scrolling with another background colour, the new line gets

the current ANSI colour (ugly: the BIOS copies the colour of the bottom left

corner)

ANSI 1.00, ?

Files

-----

AMIS.MAC	AMIS programming macros, included by RxANSIT.ASM

CMMACROS.MAC	Some of my programming macros, included by RxANSIT.ASM

make.bat	Very simple make script, assembles RxANSIT.ASM with NASM

RxANSI.ASM	Source of resident RxANSI code, included by RxANSIT.ASM

RXANSI.COM	Binary RxANSI file with default options and English messages

RxANSI.DOC	This file

RxANSI.LST	Assembling list file of RXANSI.COM

RxANSI.MAP	Assembling map file of RXANSI.COM

RxANSIT.ASM	Source of transient RxANSI code

Contact

-------

Write suggestions or bug reports to: <cm -AT- bttr-software.de>

Also try http://wwww.bttr-software.de/